MCG-2 usage note for V1.06, and later ROM software.   2-9-98

The DP (DISPLAY PAGE) command briefly blanks the screen when performing a

page update.  In many situations this is not a problem, and when changing

pages the blanking is generally not noticeable.  The DP command is meant

primarily for changing pages, however the DP command may be used at any time,

without restriction.

When updating the screen to the same page, the UP (UPDATE PAGE) command is

usually used, however in some situations (changing page position, or changing

character size) sync may be briefly overwritten by the use of this command.

To avoid this, use the DP (DISPLAY PAGE) command to change pages, change

character size, and change the page position.  The UP command may be safely

used to change pages if the new page has the same character size and has the

same page position.

The SR (SET RESPONSES) originally had two parameter, 0 and 1.  The command

now has a new parameter, 2.  Using "SR,2", after receiving a line of data,

the MCG-2 will *only* respond with a ">" prompt.  The purpose of this is to

allow for faster data transfers at 9600 baud (although you can use other baud

rates).  The ">" character acts as a line pacing character.  When the MCG-2

receives a line of data, after processing that data and when the unit is ready

to receive more data, the unit will send a ">" character.  At this time the

MCG-2 is ready for another line of data.  This method of data tranfer is

compatible with most terminal programs that support ASCII data tranfers.

Included on this disk is a file call "MCG.TXT" which is the factory sample

pages.  This file may be loaded into the MCG-2 with a terminal program

via the ASCII data transfer.

If for some reason your software should miss the ">" character (line

noise, etc.), the MCG-2 will not send another ">" character.  Your software

should time out and try to resend the line of data in this case.  Also

the first line sent should be a blank line (only a carrage return/line feed).

After sending the blank line you should look for the ">" character.  This has

two purposes, first it will clear the MCG-2 serial buffer, and second, you can

confirm that the MCG-2 is in responce mode 2.