When SET RESPONSES (SR) is set to 2, the MCG-2 will ONLY send a ">" prompt

(as verses "COMMAND" followed by ">") after receiving a line of data.  The

purpose of this is to allow for faster data transfers at 9600 baud (although

you can use other baud rates as well).  The ">" character acts as a line pacing

character.  When the MCG-2 receives a line of data, after processing that data

the unit will send a ">" character.  At this time the MCG-2 is ready for

another line of data.  This method of data tranfer is compatible with most

terminal programs that support ASCII data tranfers.

How to send data to the MCG-2 from a terminal program: In your terminal

program, set up the ASCII transfer to use a line pacing character of ">"

(ASCII code 62 decimal).  Set for no delay between lines or characters.

Before starting the ASCII transfer, send the following command by hand:

WK,01

SR,2

The file sent to the MCG-2 will be a simple ASCII file of MCG-2 commands,

one command per line, no blank lines.  In using the Write Line (WL,x) command

in a flle, be certain to send a complete line of data (the command plus 20

data characters), including trailing spaces.  Also note that in response

mode 2, no error messages will be sent if incorrect data is sent to the unit.

Lines with incorrect data will be ignored and their actions will not be

performed.

If you have the RTC option installed in your MCG-2, the Real Time Clock may

seem to stop updating durring high speed data transfers.  This is normal

and is done to allow the processor more time to process incomming data at

9600 baud.  The RTC will comtinue updating about 1/3 of a second after the

data transfer is over.  In most cases this will not be a problem as a data

transfer only takes about 3 seconds, at 9600 baud, to load all of the 10

buffer pages.

If for some reason your software should miss the ">" character (line

noise, etc.), the MCG-2 will not send another ">" character.  Your software

should time out and try to resend the line of data in this case.  Also

the first line sent should be a blank line (only a carrage return/line feed).

After sending the blank line you should look for the ">" character.  This

has two purposes, first it will clear the MCG-2 serial buffer, and second,

you can confirm that the MCG-2 is in responce mode 2.